0.8mm Pitch FBGA - 0.1 Trace

| Placement Grid | Routing Grid | Trace Width | Trace Space | Via Space | BGA Pad Size | BGA VIA Name | Route VIA Name |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 0.5, 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.4 | VIA45-20-70 | VIA70-35-100 |

0.8mm Pitch FBGA – 0.125 Trace

| Placement Grid | Routing Grid | Trace Width | Trace Space | Via Space | BGA Pad Size | BGA VIA Name | Route VIA Name |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 0.5, 0.1 | 0.05 | 0.125 | 0.125 | 0.1125 | 0.4 | VIA45-20-70 | VIA65-30-90 |

1.0mm Pitch PBGA – 0.1 Trace

| Placement Grid | Routing Grid | Trace Width | Trace Space | Via Space | BGA Pad Size | BGA VIA Name | Route VIA Name |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 0.5, 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.5 | VIA50-25-75 | VIA70-35-100 |

1.0mm Pitch PBGA – 0.15 Trace

| Placement Grid | Routing Grid | Trace Width | Trace Space | Via Space | BGA Pad Size | BGA VIA Name | Route VIA Name |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 0.5, 0.1 | 0.05 | 0.15 | 0.15 | 0.15 | 0.5 | VIA55-25-75 | VIA65-30-90 |

1.0mm Pitch PBGA – 0.125 Trace

| Placement Grid | Routing Grid | Trace Width | Trace Space | Via Space | BGA Pad Size | BGA VIA Name | Route VIA Name |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 0.5, 0.1 | 0.05 | 0.125 | 0.125 | 0.1375 | 0.5 | VIA55-25-75 | VIA70-35-100 |

1.0mm Pitch PBGA – 0.2 Trace

| Placement Grid | Routing Grid | Trace Width | Trace Space | Via Space | BGA Pad Size | BGA VIA Name | Route VIA Name |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 0.5, 0.1 | 0.1 | 0.2 | 0.2 | 0.15 | 0.5 | VIA50-25-75 | VIA70-35-100 |